- PROPOSED METHODOLOGY

3.1. Microgrid Model

The proposed methodology formulates microgrid power allocation as an optimization problem in which the available electrical supply must be distributed among multiple loads according to their demand and priority. The objective is to minimize the weighted unmet demand while ensuring that the total allocated power does not exceed the available supply.

Let a microgrid contain M loads. The demand of load i is represented by Di, its priority weight by wi, and the total available power supply by S. A higher value of wi indicates that the corresponding load has greater priority.

For continuous power allocation, the decision variable xi represents the amount of power allocated to load i. The optimization problem is expressed as:

Objective function:

minJcontx=i=1MwiDi-xi

Subject to the constraints:

i=1Mxi≤S

0≤xi≤Di,  i=1,…,M

The objective function minimizes the weighted unmet demand, while the constraints ensure that the allocated power remains within the available supply and the demand of each load.

For the quantum optimization experiments, an indivisible feeder-switching formulation is also considered. In this formulation, each feeder is either supplied completely or disconnected. Let bi∈0,1 denote the switching decision, where bi=1 indicates that feeder i is supplied and bi=0 indicates that it is disconnected.

The objective function becomes:

minJindivb=i=1MwiDi1-bi

Subject to:

i=1MDibi≤S

This formulation maps directly to the classical 0--1 knapsack problem, making it well-suited for evaluating the Quantum Approximate Optimization Algorithm (QAOA) on resource-constrained load-allocation decisions.

3.2. Decision Variables and Model Separation

The methodology separates continuous power allocation from the indivisible feeder-switching problem. Continuous allocation is handled entirely by a classical greedy fractional-knapsack algorithm, which assigns power continuously according to load demand and priority while respecting the available-supply constraint. No continuous allocation variable is discretized for quantum processing, and no fixed 10 kW chunk representation is used. QAOA is reserved strictly for the indivisible 0/1 feeder-switching problem, in which each feeder is either fully connected or disconnected. The binary decision variable for feeder i is defined as z_i ∈ {0, 1}; this combinatorial formulation is encoded as a QUBO and evaluated using QAOA.

3.3. Conversion of the Optimization Problem into QUBO

The Quadratic Unconstrained Binary Optimization (QUBO) formulation converts the constrained optimization problem into a quadratic objective function involving binary variables.

The general QUBO objective is expressed as:

Qy=c0+jajyj+j<kajkyjyk

where yj∈0,1 are binary decision variables, c0 is a constant term, aj represents the linear coefficients, and ajk represents the quadratic interaction coefficients.

Initially, the coefficients of the original optimization problem are converted into integer values using an appropriate scaling procedure. The coefficients are then reduced by their greatest common divisor (GCD), where applicable, to obtain smaller equivalent integer coefficients.

The inequality constraint on the available power supply is incorporated into the objective function using a penalty formulation. The resulting penalized objective can be written in general form as:

Qy=Jscaledy+Pjcjyj-Sscaled2

where Jscaledy is the scaled objective function, cj represents the corresponding power-consumption coefficient, Sscaled is the scaled supply limit, and P is a penalty coefficient.

The penalty term discourages solutions that violate the power-supply constraint. To convert the inequality constraint into an equality constraint before binary encoding, slack variables are introduced using a dyadic (power-of-two) expansion. This encoding represents the required nonnegative slack with a compact binary register and is selected specifically to conserve qubit-register size in the noisy intermediate-scale quantum (NISQ) era.

After expanding the squared penalty term and simplifying the resulting expression, the objective is represented in QUBO form. Integer decision variables are encoded into binary variables, producing a quadratic polynomial that can be evaluated by a classical optimizer and mapped to a quantum cost Hamiltonian.

The penalty coefficient P requires careful tuning: an insufficiently large value risks favoring constraint-violating solutions, whereas an excessively large penalty can distort the optimization landscape, hindering effective exploration.

3.4. Construction of the QAOA Circuit

The QUBO objective is converted into an equivalent Ising Hamiltonian using the transformation:

yj=1-Zj2

where Zj is the Pauli-Z operator acting on qubit j.

The resulting cost Hamiltonian is written as:

HC=C0I+jhjZj+j<kJjkZjZk

where C0 is a constant, hj represents the single-qubit coefficients, and Jjk represents the interaction coefficients between pairs of qubits.

QAOA uses this Hamiltonian to construct a parameterized quantum circuit. The initial state is prepared as an equal superposition of computational basis states:

|+⟩⊗N=H⊗N|0⟩⊗N

where N is the number of qubits and H denotes the Hadamard gate.

The cost unitary is defined as:

UCγ=e-iγHC

The mixing Hamiltonian is defined as:

HM=j=1NXj

where Xj is the Pauli-X operator acting on qubit j. The corresponding mixing unitary is:

UMβ=e-iβHM

The QAOA state at circuit depth p is obtained by alternately applying the cost and mixing unitaries:

UMβpUCγp⋯UMβ1UCγ1|+⟩⊗N

The parameter vectors are defined as:

γ=γ1,…,γp

β=β1,…,βp

The parameters are optimized to identify a circuit configuration that produces low-cost solutions with high probability.

3.5. Measurement and Candidate Solution Selection

After preparing the parameterized QAOA state, measurements are performed in the computational basis. Each measurement produces a binary bitstring corresponding to a candidate solution of the encoded optimization problem.

A total of 2,048 measurement shots are used for each evaluated parameter configuration. The observed bitstrings are decoded into the corresponding decision variables, and their objective values are evaluated using the original discrete optimization model.

Candidate solutions are assessed for feasibility by checking whether they satisfy the available-power constraint. Feasible candidates are then ranked according to their objective values, with lower weighted unmet demand indicating a better solution.

Candidate selection relies on evaluating the decoded objective value of each unique bitstring rather than relying solely on measurement frequency. Because high-frequency measurement outcomes do not guarantee optimal objective values—and standard post-processing filters may discard low-frequency valid solutions—all unique raw bitstrings are decoded and checked for constraint satisfaction. Deterministic tie-breaking is applied when multiple candidates have identical objective values.

This procedure provides a more reliable assessment of the quality of the solutions generated by QAOA.

3.6. Classical Optimization Loop

QAOA is a hybrid quantum-classical algorithm. The quantum circuit evaluates the parameterized state, while a classical optimizer updates the circuit parameters based on the measured objective value.

The optimization procedure consists of the following steps:

- Formulate the microgrid allocation problem and identify its decision variables and constraints.

- Convert the constrained optimization problem into a QUBO objective.

- Map the QUBO objective to the corresponding Ising cost Hamiltonian.

- Construct the parameterized QAOA circuit for a selected depth p.

- Execute the circuit simulation and obtain measurement samples.

- Decode the measured bitstrings and evaluate their objective values.

- Calculate the expected cost associated with the current QAOA parameters.

- Pass the cost to the classical optimizer, which updates the parameters γ and β.

- Repeat the circuit evaluation and parameter update until the stopping criterion is satisfied.

- Select the best feasible candidate observed during the optimization process and compare its objective value with the classical reference solution. The validated quantum statevector output (for example, [1, 0, 1, 1]) is then serialized as a JSON payload and streamed to a custom WebGL 3D digital twin implemented with the Antigravity engine, which visually maps the inferred real-time energy flow onto the corresponding physical infrastructure.

The COBYLA classical optimizer is used for the depth-sweep experiments, with a maximum of 200 function evaluations per optimization run. The optimization loop is implemented using Python-based quantum-circuit simulation tools.

3.7. Experimental Configuration

In this study, QAOA is executed using quantum-circuit simulations on a classical CPU; consequently, the experiments evaluate the optimization behavior of simulated QAOA rather than the performance of physical quantum hardware.

The experimental evaluation examines the effect of QAOA circuit depth on solution quality and computational performance. For the depth-sweep experiments, the circuit depth is varied across p ∈ {1, 2, 3}.

p∈1,2,3,4,5

Three random seeds are used:

Seeds=42,123,999

Each evaluated configuration uses 2,048 measurement shots. These settings support comparisons across different circuit depths and random initializations.

A separate benchmark evaluates indivisible feeder-switching instances. Eight scenarios containing three to six feeders are considered, with problem encodings requiring approximately five to nine qubits. Circuit depths p ∈ {1, 2, 3} are evaluated using three random seeds.

The experiments record solution quality, feasibility, runtime, and the relationship between circuit depth and optimization performance. The use of multiple scenarios and random seeds helps assess the consistency of the results across different problem instances and initializations.

3.8. Classical Benchmarking and Performance Evaluation

The QAOA results are compared with classical optimization solutions obtained using SciPy's HiGHS-based mixed-integer linear programming (MILP) solver for the discrete feeder-allocation problem.

The classical reference solution is used to evaluate the quality of the best feasible QAOA candidate. The comparison considers the following metrics:

- Objective value: The weighted unmet demand associated with the selected solution.

- Feasibility: Whether the candidate satisfies the available-power constraint.

- Optimality gap: The difference between the candidate objective and the classical reference objective, interpreted relative to the reference value where appropriate.

- Exact-optimum recovery: Whether QAOA produces a candidate matching the classical optimum.

- Runtime: The computational time required to obtain a solution.

- Ground-state probability mass: The measured probability assigned to optimal or minimum-energy states, where applicable.

For minimization problems, the objective value of the selected feasible QAOA solution is compared with the classical optimum. A zero objective gap indicates that the same optimal objective value has been recovered.

The continuous power-allocation formulation is also distinguished from the discrete QUBO experiments. Under the stated continuous model, allocation can be solved using a classical greedy fractional-knapsack approach. Therefore, QAOA is primarily evaluated on the discrete combinatorial formulation rather than presented as a replacement for an already efficient continuous optimization method.

A separate 24-hour dynamic-dispatch formulation, when considered, is solved using classical HiGHS linear programming and is not treated as a QAOA result.

3.9. Summary of the Proposed Workflow

The proposed workflow begins by modeling the microgrid and defining the load demands, priority weights, available supply, and feeder-switching variables. Continuous power allocation is solved entirely by a classical greedy fractional-knapsack algorithm; it is not discretized for quantum processing. For the separate indivisible feeder-switching problem, the binary constrained formulation is converted into a QUBO, with dyadic slack-variable encoding used to convert the supply inequality into an equality while conserving qubit-register size. The QUBO is mapped to an Ising cost Hamiltonian, and a parameterized QAOA circuit is constructed for p ∈ {1, 2, 3}. The circuit is simulated, measurement outcomes are decoded and checked for feasibility, and COBYLA updates the circuit parameters for at most 200 function evaluations per run.

The best feasible candidate is compared with the classical reference solution using objective value, feasibility, optimality gap, optimum-recovery rate, and runtime. The validated quantum statevector output (e.g., [1, 0, 1, 1]) is serialized into a JSON payload and streamed to a custom WebGL 3D digital twin built with the Antigravity engine. The digital twin maps the selected feeder states and corresponding real-time energy flow onto the modeled physical infrastructure, providing a visual representation of the optimization output.

Because the study is conducted using classical quantum-circuit simulation, the results demonstrate the behavior of simulated QAOA on the selected optimization instances and do not establish a quantum advantage over classical optimization methods.